
Material Safety Data Sheet
	Section1-Chemical Product & Company Identification

	产品中文名称
	天然橡胶  

	产品英文名称
	Natural rubber  

	中文名称2
	NA

	英文名称2
	NR

	技术规范编号
	1315

	CAS 号.
	9003-31-0 

	分子式
	C14H28O2

	分子量
	228.37100

	Section2-Composition / Information on Ingredients

	成分
	content
	CAS No.

	Natural rubber (NR)
	            48%
	9003-31-0 

	   calcium carbonate
	            20%
	471-34-1

	     Titanium dioxide                                
	            10%
	1317-80-2

	White carbon black
	            5%
	14464-46-1

	Stearic acid
	            4%
	57-11-4

	     TT
	            3%
	137-26-8

	S
	            5%
	63705-05-5

	AC
	            3%
	123-77-3

	DM
	            2%
	120-78-5

	Sectioin3-Hazards Identification

	Risk category风险种类
	NA

	International dangerous goods no.国际危险品号
	NA

	Danger level危险级别
	NA

	Health hazard健康危害
	This material has stimulation to eyes and skin.本品对眼睛和皮肤具有刺激性

	Environmental hazard环境危害 
	NA

	Combustion hazard燃爆危害
	This material is inflammable and stimulated本品易燃具刺激性

	Section4-First Aid Measure

	Skin contact皮肤接触
	Remove contaminated clothing. Wash contaminated skin with soap and water. 脱去被污染的衣服。用肥皂和水清洗被污染的皮肤。

	Eye contact眼部接触
	Rinse with plenty of water, then consult oculist

用大量的水冲洗,然后咨询眼科医生

	Inhalation吸入
	remove to fresh air. If symptoms persist, obtain medical attention. 
移至新鲜空气。如果症状持续,咨询医疗。

	Ingestion摄入
	Induce vomiting催吐


	Section5-Fire Fighting Measures 消防措施

	Special exposure hazards特殊暴露危害
	Formation of carbon monoxide, carbon dioxide

形成碳氧化物

	Protection equipment

保护设备

Extinguishing media适用灭火剂
	Wear respiratory protection equipment. 灭火介质，戴呼吸防护设备。
H2O, CO2, dry chemical powder水、二氧化碳、干粉

	Section6-Accidental Release Measures泄露应急处理 

	Personal protection

个人防护
	Wear respiratory protection equipment戴呼吸防护设备

	Method of cleaning up

清理的方法
	Sweep up spilled material for use or disposal. Good house keeping must be maintained to avoid potential slipping problem. 清理掉溢出的材料使用或处理。必须保持良好的内务工作,以避免潜在的滑动问题。

	Section7-Handling and Storage保存和存储

	Procedure for handling

处理过程
	Avoid all personal contact, including inhalation.

Wear protective clothing when risk of exposure occurs.

Use in a well-ventilated area.

Prevent concentration in hollows and sumps. 

Empty containers may contain residual dust which has the potential to accumulate following settling. Such dusts may explode in the presence of an appropriate ignition source.

Do NOT cut, drill, grind or weld such containers.

In addition, ensure such activity is not performed near full, partially empty or empty containers without appropriate workplace safety authorisation or permit.

	Storage requirements

存储要求
	Store in original containers. 
避免所有个人接触,包括吸入。如有暴露风险发生需穿防护服时。
在通风良好的区域使用。
防止在洞里和污水坑浓缩。
空容器可能含有残留的灰尘有可能积累沉淀。这些粉尘遇适当的着火源可能会引发爆炸。
Keep containers securely sealed.

Store in a cool, dry, well-ventilated area.

Store away from incompatible materials and foodstuff containers.
保持容器的安全密封。
储存在阴凉、干燥、通风良好的区域。
存储时远离不相容的材料和食品容器


	Section8-Exposure Controls and Personal Protection 接触控制和个人保护

	OEL
	N/A

	TLVTN
	N/A

	TLVWN
	N/A

	Monitoring method

监测方式
	N/A

	Engineering control

工程控制
	Closed operation. Provide good natural ventilation condition.

封闭操作。提供良好的自然通风条件。

	Respiratory protection

呼吸防护
	Wear air-supplied respirator.戴防护口罩

	Eye眼部
	Safety glass with side shields.带防护板的安全眼镜

	Body身体
	Wear work clothes戴工作手套

	Hand手部
	Wear rubber gloves.戴橡胶手套

	Other其他
	Smoking is prohibited in working site. Keep good hygienic habit. 

	Section9-Physical Properties 

	Major constituent
	Natural rubber   

	Appearance
	White yellow block

	Melting point(℃)
	95℃

	Boiling point(℃)
	122-144℃

	Relative density(water=1)
	0.89 g/cm3

	Relative vapor density (air=1)
	NA

	Saturated vapor pressure (kPa)
	NA

	Heat of combustion (kJ/mol)
	NA

	Critical temperature(℃)
	NA

	Critical pressure(MPa)
	NA

	Flashing point(℃)
	NA

	Ignition temperature(℃)
	NA

	Upper explosive limit%(V/V)
	NA

	Lower explosive limit%(V/V)
	NA

	Solubility
	NA

	Main application主要应用
	Use may require material be molten. Molten or heated material may be compounded, moulded or extruded. Used in shoes materials, hot melt adhesives, coatings and fabricated products.
使用可能需要物质熔融。熔融或加热材料可能加剧,模压或挤压。用于鞋材料,热熔粘合剂、涂料和制造产品。

	Section10-Chemical Stability化学稳定性

	Prohibited mixture

禁止混合
	NA

	Conditions to avoid
	Hydrocarbons, halogenated hydrocarbons, carbon disulfide, ethers and higher fatty acids, concentrated strong acids and highly oxidizing potassium permanganate, potassium heavy acid, toluene, xylene, etc.

烃、卤代烃、二硫化碳、醚和高级脂肪酸、浓强酸和氧化性强的高锰酸钾、重酸钾、甲 苯、二甲苯等。

	Section11-Toxicological Information

	Toxicity
	NA

	Sensitive
	NA

	Amyctic
	NA

	Section12-Ecological Information

	Eco-toxicological
	NA

	Other deleterious effects
	NA

	Section13-Disposal Considerations

	Disposal method
	Disposal through controlled incineration, melt-out, granulation or authorized waste dump in accordance with national and local regulations.

	Section14-Transport Information

	Code of dangerous goods
	NA

	UN no.
	NA

	Packaging category
	Z01

	Packing method
	NA

	Transport notes
	The package should be complete before transport and loading should be secure. The package should be anti-sun, rain and anti-high temperature. Do not mix with oxidant, alkali, edible chemical. Clean the vehicles after transportation. 

	Section15-Regulatory Information

	No data
	

	Section16-Other Information


The information supplied has been based upon the current level of information available, for the purpose of specifying the requirements regarding environment, health and safety in conjunction with the product. They are not to be interpreted as a warranty for specific product characteristics. We take no responsibility for inappropriate use, processing and handling by purchasers and users of the products.

基于当前可利用的信息水平提供信息,旨在对有关环境,健康和安全有特定需求的产品。他们不能被解解读为具有某种特定产品特性的保证。我们对购买者的不当使用、加工和处理产品不负责任。

1/(2)

